- BCM5700/BCM5701

- Diagnostic Script File User’s Guide

- Revision 1.0  Date 2/08/02

- Supports B57diag Version: 2.29 or later

- Prepared by: Tak Tomita

- B57diag supports script files. The concept is similar to DOS’s batch file. User can put a sequence of commands in a text file to be executed and invoked by ‘do’ command from the engineering prompt. A file autoexec.do is automatically executed if it is present in the working directory. In addition, user can enter the DOS command prompt parameter –do <filename> to execute the file other than autoexec.do upon start up. This works for both engineering mode and manufacturing mode.

To make script file more powerful and flexible, the ‘if’ and ‘while’ statements are supported. In addition, the variable assignment is also supported. This document describes the usage of the script file.

# Syntax

Each command in the script file uses exact one line in the script text file. An empty line is ignored. Please refer to b57diagman.doc for available commands.

## The ‘do’ command

The script file is execute by the ‘do’ command.

0>do [option] <filename> [with <parameter1>, …]

Option:

-e 		Echo command

-p<#>		Pause number of millisecond between each command

-c 		Continue on Error

User can pass up to 10 parameters. Each parameter is evaluated and assigned to the variable defined by the command ‘parameter’ in the script file.

parameter <variable1>, <variable2>, ….

The number of passing parameter must be equal or more than the number of variable defined in the parameter statement.

Each parameter variable is created locally in the script file. Upon exit the file script file, the variable is purged.

Example:

A file ‘myscript.do’ contains the following text:

parameter loop

while (loop > 0)

printf(“I am in loop %d\n”,loop)

sleep 1000

loop = loop –1

if (loop=3)

break

endif

endwhile

return

The script can be started by

0>do myscript.do with 10

The output will be:

I am in loop 10

I am in loop 9

I am in loop 8

I am in loop 7

I am in loop 6

I am in loop 5

I am in loop 4

0>

Each do script can return a value. The result will be printed out the console. For example,

parameter loop

if (loop > 20)

return “I think it will take too long”

endif

while (loop > 0)

printf(“I am in loop %d\n”,loop)

sleep 1000

loop = loop –1

if (loop=3)

break

endif

endwhile

return loop + 10

0>do myscript.do with 10

The output will be:

I am in loop 10

I am in loop 9

I am in loop 8

I am in loop 7

I am in loop 6

I am in loop 5

I am in loop 4

13

0>

0>do myscript.do with 50

“I think it will take too long”

Comments:

A line starts with a ‘;’ will be treated as comment line. The leading spaces are allowed.

Variables:

Current script file supports 3 types: integer (u32), string, and logic

Name must start with alpha character and can have alpha or numeric afterwards.

For example: myname123

The maximum name length is 16 characters. Anything over 16 will be truncated

integer: 0 - 0xffffffff

string: A string with start and end with double or single quote

logic: true or false

true and false are reserved words for logic

Scope:

The do command or function calls can be nested or called recursively. Any variable created in parent do is visible from child do procedures. When the child do terminates, the variables created in that process is also get terminated and purged. If the child creates a same variable name from the parent, a local copy is created. It will not affect the parent variable.

## Variable Assignment:

<variable name> = <expression>

## Expression Evaluations

The evaluation always go from left to right, doesn't matter what the operation is.

For example,

Bbb = 26

ccc = bbb + 10 * 2

The result for ccc is 72

If multiplication needs to be done first before addition, a set of parenthesis needs to be need as follow:

ccc = bbb + (10*2)

Now ccc is 46

## The if Statement:

if <logical expression>

....

[elseif <logical expression>]

.....

[else]

.....

endif

The while statement:

while <logical expression>

...

[break]

...

endwhile

The if and while statements can be nested, and of course, can be used together.

Built in Functions:

Currently, b57diag supports three functions: read(), write(), and printf()

read(string devType, integer address)

string: specify the type of read, this one byte string specifies what kind of read. This is same as b57diag read/write defines.

'#'    mac register

'*'    sram

'~'    vpd

'!'     config space (32 bit)

'S'     config space (16 bit)

'X'     config space (8 bit)

'$'     nvram

'^'     indirect memory access

'm'    mii registers

'l'     direct host memory (32)

's'      direct host memory(16)

'x'     direct host emory (8)

write(string devType, integer address, integer data)

string: specify the type of write, this one byte string specifies what kind of write. This is same as b57diag read command’s device type character.

printf(string format [, parameter1 [, parameter2, .... ]])

same as C printf functions; however, can only pass up to max. of 10 parameters.

User Defined Functions:

Support up to max 10 parameters.

Name must start with alpha characters and can have alpha and numeric characters afterwards. It is the same definition as variable names.

Functions must have a set of parenthesis after the name.

For example: 	myfunction()  -- function with no parameters

MyFunc123(10) – function with a integer parameter.

The user function is just another script file, however, it must has the file extension ‘.fnt’

Each function script file must return a value with return statement.

For example:

We create a file called double.fnt contains the following two lines

parameter number

return number * 2

In engineer prompt, we type:

0>myVar = 5

0>newVar = double(myVar)

0>printf(“%d\n”,newVar)

10

0>

It prints the result 10 to console.

# Operators:

integers:

+        addition

-         subtraction

*         mulitiplication

/         division

and     bitwise and

xor     bitwise excusive or

or       bitwise or

not     bitwise (unary operator)

>>     shift right

<<     shift left

=       equal

>       greater

<       less then

>=     greater or equal

<=     less or equal

!=      not equal

strings

=       equal

>       greater

<       less then

>=     greater or equal

<=     less or equal

!=      not equal

and    concatenate the strings

logic

and     logic and

or       logic or

xor     logic xor

not     logic not (unary operator)

## Examples

; declear a numeric variable called abc. (in hex format)

abc =  0x10

; declear another variable called bbb. (decimal format + hex format will become hex format)

bbb = 10 + abc

; now bbb is 26 (0x1a)

; the evaluation always go from left to right, doesn't matter what the operation is

ccc = bbb + 10 * 2

; ccc is 72

; if multipication needs to be done first before addition, a set of parenthesis needs to be added

ccc = bbb + (10*2)

; now ccc is 46

; reads data from pci config space 0 and save it to variable 'deviced'

deviceid = read('!',0)

; command 'var' will display all the variables

var

;

if deviceid != read('$',0xa0)

printf("Device Id is not the same as NVRAM setting, firmware probly is not running\n")

printf("register reports %08X and NVRAM is %08X\n",devieid, read('$',0xa0))

elseif read('$',0xa0) = 0

printf("NVRAM contains invalid device id\n")

elseif false

never comes to this here, since the condition is false....

anything in here is ignored.....

therefore, it will not be an error.... :)

so you can type anything you can.

adlkd;adlkjsadlj;sdjlfds

adfkdfkljksadjlkfda

in fact, you can use this method for comments...

else

printf("this is in else block\n")

endif

counter = 1

while counter < 10

printf("trial %d\n",counter)

; poll bit 8 of register 404 to be one

if (read('#',0x404) and 0x100) = 0x100

break

endif

sleep 100

counter = counter + 1

endwhile

; create a beep sound

beep